PLAXIS 3D Lesson 3:

Learning Objective: This lesson is intended to introduce you to determining expected settlement caused by any specific surface load and help you understand the effects of water level on square footing settlement.

- Open the saved project (Lesson 2: bearing capacity of square footing)

- Click on the File tab in the toolbar, and then click Save project as… to create a copy of the current file and apply the new changes in the new file.

- In the opened window, choose a name to be saved with, for example, type in: bearing capacity of square footing-load.

- Click Save.

Task 1: Apply Surface Load

You can apply a specified load and calculate displacement corresponding to the applied load:

- Click on the Structures mode. This is specified with a Red circle in the figure below.

- To apply load instead of prescribed displacement, select the Model explorer section, click on the + sign next to Surface displacement, right-click on Surface Displacement, and then click Delete.

- Click on the volume object in the 3D model (the selected object will change to red color), then right-click on the selected volume, and press Hide.

- Select the plate object in the 3D model, right click on the selected plate, select Create, and then click Create surface load.

- To assign a specific load, in the Model explorer section, click on the + sign next to Surface loads, then click the + sign next to SurfaceLoad_1, enter σz  value: -1000 kN (note: - before the number indicates its downward direction).

Task 2: Create the Mesh and Generate Model

- Go to Mesh mode.

- Click on the Generate mesh icon. In the opened window, click on the drop-down arrow and choose Very coarse for Element distribution.

- Click OK.

- Click on Select points for curves to select a point for calculations.

- In the opened window, enter -0.5 m in the Z box (this is the footing depth defined before), click on Search closest, among the found nodes, and check the closest one to the specified position. Make sure to select a point on the plate. In the found notes section, under the “Data from” column, the selected point should be on a Plate (specified with a Green circle in the figure below)

- Click Update.

Task 3: Define phases

- Go to the Staged construction mode to define construction phases.

- In the Phases explorer, click Edit phase.

- In the opened window, some phases should be already defined from the previous example.

- In the opened window, go to the Phase_2. Make sure “Reset the displacement to zero” in the Deformation control parameters is checked.

- Go to Phase_3. Make sure “Reset displacement to zero” in the Deformation control parameters is checked.

- Click OK.

- Click on the Initial phase in the Phases explorer, then, in Model explorer, make sure “Soils” is checked (both Soil_1 and Soil_2 have to be checked).

- Click on the Phase_1 in the Phases explorer, then, in the Model explorer, uncheck the box next to Soil_2.

- Click on the Phase_2 in the Phases explorer, then, in the Model explorer, check the box next to Plates.

- Click on the Phase_3 in the Phases explorer, then, in the Model explorer, check the box next to Surface loads.

Task 4: Perform Calculations and View Results

- Click Calculate.

- After finishing the process, select View calculation Results.

- To observe the result of total displacement in Z direction, click on the Deformation tab in the toolbar, then Total displacements, and select uz.

- To plot a load vs. settlement graph, in the PLAXIS output window, click on Curves manager.

- In the opened window, click on New to define the desired curve.

- To see a desired cross-section of the model, click on the Vertical cross section icon. You can draw a section line by clicking on two points of a section, as shown in the figure below, or by adding each point’s coordinates in the opened window.

- After creating a section view, you can go to the View tab, select Viewpoint, and then choose a direction to see the section.

- In the opened window, by clicking on the Right view icon, you can see the displacement values.

- The view can be adjusted by Pan camera (for moving model) and Rotate camera (for rotating model).

- Click on the icon with the arrows as shown in figure below to see the displacement movement direction, the upward direction indicate soil heave due to foundation settlement.

- To draw Load vs. Displacement curve, in the X-Axis section, keep the dashboard on Project, in the Step subsection, Choose “ƩMstage”. In the Y-Axis section, click on the drop-down arrow under the y-axis and choose the node you defined in previous sections. Then, click on the + sign next to Deformation, then click on the + sign next to Total displacement, and choose “uz”.

- Click OK.

.

- In the opened window, you can see the Load-Displacement plot.

- To delete data from undesired phases, right-click on the plot sheet and go to Settings.

- In the opened window, go to the Node tab, select Phases, make sure to uncheck all the phases except Phase_3.

- Click OK.

- To export data for further analysis, click on the Table icon (shown in figure below).

- Right-click on parameters specified with Red circles in figure below, and click copy.

- Open an Excel file and paste each specified column.

- Multiply the ƩMstage column by the applied load value (1000).

- Multiply the u_z column by -1 to get positive numbers and by 1000 to convert it to millimeters.

- In the Excel file, click on the item specified with a red circle in the figure below to select the entire sheet. Click on the drop-down arrow to choose cell format, and choose General.

- Plot the Load-Displacement curve in the Excel file and save it for later.

Task 6: Define Cohesive Soil and Analyze Water Level Effects

To see the effect of groundwater level on the behavior of the foundation, first, define a cohesive soil:

- Use “Save as” to save the project you were working on with a new name. For example: bearing capacity of square footing- water level.

- Click on Soil mode, then Show materials, and then New to define the new soil.

- In the opened window, type “Clay” in Identification, change the Drainage type by clicking on drop-down arrow and choose, “Undrained A”.

- Enter unsaturated and saturated soil densities: unsat : 16 kN/m2 , sat : 19 kN/m2

- Click on the Material tab, and enter soil mechanical soil properties:

Eref': 8000 kN/m2, Cref': 30 kN/m2  ,  ø': 15

- Click OK.

- After defining soil properties, click OK.

- In the Model explorer, click on the + sign next to Stratigraphy, click on the + sign next to Borehole_1, and change the Head value to -0.5 m.

- Click on the + sign next to Soils and make sure to change both Soil_1 and Soil_2 Materials to “Clay” by clicking on the drop-down arrow next to its box.

- Perform Task 3, Task 4, and Task 5 again and plot the Load-Displacement curve in Excel.

- Change the head to -5 m, then perform Task 3, Task 4, and Task 5 again and plot the Load-Displacement curve in the same excel file as above.

- The figure below shows the Load-Displacement plot for two different water levels in the defined cohesive soil: